# DECOUPLING

INTRODUCTION:

Permanent Magnet Synchronous Motors (PMSMs) are widely used in electric vehicles due to their high efficiency, torque density, and controllability. However, PMSM dynamics are nonlinear because the d-axis and q-axis currents affect each other. This creates a coupling effect that makes precise torque and flux control difficult.

To overcome this, a decoupling block is used. My part of the project implements exactly this decoupling logic.

WHY DECOUPLING IS NEEDED

• PMSM has two control currents:

– id controls magnetic flux.

– iq controls torque.

• In an ideal scenario, id and iq should work independently.

• But in real motors, changing id unintentionally affects iq and vice versa.

• This coupling leads to errors in torque production and unstable control.

• Decoupling removes these unwanted cross-interactions.

UNDERSTANDING THE d-q REFERENCE FRAME

• ABC currents (three-phase) are sinusoidal → difficult to control.

• Clarke and Park Transformation convert these into:

– d-axis: aligned with rotor flux.

– q-axis: perpendicular to rotor flux.

• In dq-frame:

– Motor behaves like a DC system.

– Control becomes simple.

– Flux and torque can be independently regulated.

EQUATIONS BEING IMPLEMENTED

The PMSM voltage equations in dq-frame:

vd = Rs * id + Ld * did/dt − ωs * Lq * iq

vq = Rs * iq + Lq * diq/dt + ωs * (Ld * id + φ)

The terms highlighted below represent coupling:

• − ωs * Lq * iq   ⟶ coupling term in vd

• + ωs * Ld * id   ⟶ coupling term in vq

• + ωs * φ        ⟶ back EMF term in vq

The PID controller provides linear voltages vdl and vql.

The decoupler adds back the missing coupling terms.

MATLAB FUNCTION IMPLEMENTATION

code:

function [vd,vq]= decouple(ws,vdl,vql,iq,id) Ld = 1e-4; Lq = 1e-3; phi = 0.23; vd = -ws*Lq*iq + vdl*Ld; vq = ws*phi + ws*Ld*id + vql*Lq;

Explanation:

• ws = electrical speed of PMSM.

• vdl, vql = linear voltages created by PID controllers.

• iq, id = currents from motor.

• Ld, Lq = inductances.

• φ = permanent magnet flux.

What the function does:

1. Computes vd coupling term: −ω * Lq * iq

2. Computes vq coupling terms: ωφ + ω * Ld * id

3. Adds them to the linear voltages.

4. Outputs corrected voltages vd and vq.

BLOCK DIAGRAM FLOW (SUBSYSTEM)

Inputs:

• ws (speed)

• VdqLin (linear voltages: vdl, vql)

• Idq (currents: id, iq)

Steps:

1. Demux VdqLin → vdl & vql

2. Demux Idq → id & iq

3. MATLAB Function “decouple” computes:

– corrected vd

– corrected vq

4. Mux → final Vdq vector

5. Output to PMSM block

WHY THIS BLOCK IS IMPORTANT

• Ensures torque produced = torque demanded.

• Removes unwanted motor dynamics.

• Makes control smooth, stable, predictable.

• Allows EV to accelerate/decelerate safely.

• Essential for field-oriented control (FOC).